SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL2-DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | Prathipa S | Reg.No.: | 192521419 |
| Department: | Btech-IT | Date: | 03-08-2016 |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

Code of Conduct:

I Prathipa S(192521419)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctnes s of Debugged Solution (25%) | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

Question 1. Signed Integer Overflow in 2's Complement Addition (+120 and +50, 8-bit)

1. Identification of the Bug

The program adds two positive 8-bit signed numbers, +120 and +50. Their true mathematical sum is 170. However, an 8-bit signed (two's complement) register can only represent values from −128 to +127. Since 170 is outside this range, the program cannot produce a correct result — the bug is a signed overflow that the program is failing to detect.

2. Debugging Approach & Analysis (Binary Trace)

Step 1 — Convert both operands to 8-bit two's complement:

| Value | Binary (8-bit) |
| --- | --- |
| +120 | 0 111 1000 |
| +50 | 0 011 0010 |

Step 2 — Add bit by bit (LSB → MSB), tracking the carry out of each position:

| Bit position | b7 b6 b5 b4 b3 b2 b1 b0 |
| --- | --- |
| +120 | 0 1 1 1 1 0 0 0 |
| +50 | 0 0 1 1 0 0 1 0 |
| Carry generated | 1 1 0 0 0 0 0 0 (carry OUT of each column, right→left) |
| Result | 1 0 1 0 1 0 1 0 |

Reading the result bits gives 1010 1010. As an unsigned value this is 170 (correct arithmetically), but because bit 7 (the sign bit) is 1, the hardware interprets it as a signed number: 10101010 = −86 in two's complement.

Verification: 120 + 50 = 170 (expected). The register instead reports −86. The 256-wide wraparound confirms it: 170 − 256 = −86, exactly the value produced — i.e., the sum silently wrapped around the representable range.

3. Root Cause

Two positive numbers were added and produced a negative result. This is only possible when the result has overflowed the signed range. The specific hardware-level cause is that the carry INTO the sign bit (bit 7) differs from the carry OUT of the sign bit:

| Carry into bit 7 (from bit 6) | 1 |
| --- | --- |
| Carry out of bit 7 | 0 |

Because these two carries differ (1 ≠ 0), an overflow has occurred. This is the standard hardware overflow-detection rule.

4. Corrected / Verified Solution

Overflow (V flag) should be computed as:

Overflow detection rule V = Cin(sign bit) XOR Cout(sign bit) Equivalently, using operand and result signs: V = 1 if (sign(A) == sign(B)) AND (sign(Result) != sign(A))

Applying this rule: sign(+120)=0, sign(+50)=0 (same sign), sign(result)=1 (different) → V = 1 → overflow correctly flagged. The program should raise an overflow exception/flag instead of silently returning −86, and either (a) reject the operation, (b) saturate to the maximum representable value (+127), or (c) automatically promote the computation to a wider register (16-bit or 32-bit) where 170 fits without overflow.

5. Optimization / Hardening Technique

- Add a dedicated hardware overflow flag (V) computed via the Cin XOR Cout rule at the ALU's sign-bit stage — this is O(1) extra logic, no performance cost.

- For software running on fixed-width registers, use saturating arithmetic intrinsics (e.g., saturate to INT8_MAX) when overflow is detected, so results degrade gracefully instead of wrapping silently.

- Where the value range is known to exceed 8 bits, use a wider data type (16/32-bit) from the start — this removes the overflow risk at the cost of extra memory, a standard performance/safety trade-off.

6. Testing & Verification

Test cases to confirm the fix:

| Test case | Expected behaviour |
| --- | --- |
| 120 + 50 | Overflow flag = 1; result handled per chosen policy (reject/saturate/widen) |
| 100 + 20 | No overflow (sum = 120, within range); flag = 0 |
| -120 + -50 | Overflow flag = 1 (negative overflow, wraps to positive) — confirms rule also catches negative-side overflow |
| -100 + 50 | No overflow; mixed-sign addition can never overflow |

Question 2. Ripple Carry Adder Delay Bottleneck in a Real-Time System

1. Identification of the Bug / Bottleneck

The processor uses a Ripple Carry Adder (RCA): n single-bit full adders chained together, where each stage's carry-out feeds the next stage's carry-in. In a real-time system this creates unacceptable latency because every bit position must wait for the carry from every earlier position before it can produce its final sum.

2. Debugging Approach & Analysis

Tracing the carry path bit by bit exposes the problem: each full adder has an intrinsic gate delay (commonly modelled as ~2 gate delays, Δ, to produce a carry-out). Because carry Ci+1 depends on Ci, the total worst-case delay for an n-bit RCA is:

Ripple Carry Adder delay T(RCA) = n × Δ (linear in the number of bits, O(n))

For a 32-bit adder this means the final sum bit cannot be trusted until the carry has rippled through all 32 stages — roughly 32× the delay of a single full adder. In a real-time system with a fixed clock period, this serial dependency directly caps the maximum clock frequency, which is the observed bottleneck.

3. Root Cause

The delay is structural, not a coding error: it is inherent to how carries propagate serially through a ripple-carry chain. No amount of software-level optimization fixes it — the adder architecture itself must change.

4. Optimized Solution — Carry Look-Ahead Adder (CLA)

A CLA removes the serial dependency by computing every carry directly from the original inputs, in parallel, using two auxiliary signals per bit:

| Signal | Definition |
| --- | --- |
| Generate, Gi | Gi = Ai · Bi (this stage produces a carry regardless of Ci) |
| Propagate, Pi | Pi = Ai ⊕ Bi (this stage passes an incoming carry through) |

Each carry can then be expanded algebraically without waiting for the previous stage:

Carry look-ahead equations C1 = G0 + P0·C0 C2 = G1 + P1·G0 + P1·P0·C0 C3 = G2 + P2·G1 + P2·P1·G0 + P2·P1·P0·C0 …and so on — every carry is a two-level AND-OR expression of the primary inputs, computable simultaneously.

Because every Ci is derived directly (not by rippling), the critical path length no longer scales linearly with n. In practical multi-level CLA designs (e.g., 4-bit CLA blocks combined by a second-level look-ahead unit), the delay grows only logarithmically:

Carry Look-Ahead Adder delay T(CLA) ≈ O(log n) vs. T(RCA) = O(n)

5. Why This Reduces Delay

- Every carry is computed from the original A and B bits in at most 2 gate levels, instead of waiting n stages for the ripple.

- Grouping bits into 4-bit CLA blocks and cascading a second-level look-ahead unit over the blocks keeps the critical path to roughly log₄(n) block-levels instead of n single-bit levels.

- This trades additional hardware (more AND/OR gates for the look-ahead logic) for significantly higher speed — exactly the kind of area-for-speed optimization real-time systems need.

6. Testing & Documentation

Verification approach: simulate both adders (RCA and CLA) for identical operand pairs across the full input range and confirm identical sums (functional correctness is unchanged — only timing differs). Measure critical-path delay via static timing analysis or gate-level simulation for increasing bit-widths (8, 16, 32, 64) and confirm the RCA delay grows linearly while the CLA delay grows logarithmically, documenting the crossover point at which CLA's extra hardware cost is justified by the real-time deadline.

Question 3. Booth's Algorithm Producing Incorrect Results for Negative Operands

1. Identification of the Bug

Booth's algorithm multiplies signed numbers by examining pairs of bits (Qn, Q₋₁) and shifting an extended accumulator. When it fails specifically for certain negative operands (while working for positive ones), the bug almost always lies in one of three places: incorrect sign extension during the shift, an incorrectly computed two's-complement of the multiplicand, or a miscounted number of iterations.

2. Debugging Approach — Step-by-Step Trace of the Algorithm

Standard Booth's algorithm for an n-bit multiplication uses registers AC (accumulator, initialised to 0), Q (multiplier), Q₋₁ (initialised to 0), and M (multiplicand):

| (Qn, Q₋₁) | Action |
| --- | --- |
| 0 0 | No arithmetic operation — this is a run of 0s, just shift |
| 0 1 | AC = AC + M (start of a run of 1s) |
| 1 0 | AC = AC − M (end of a run of 1s) |
| 1 1 | No arithmetic operation — inside a run of 1s, just shift |

After the add/subtract decision, the algorithm performs one arithmetic right shift (ASR) across the combined (AC, Q, Q₋₁) register, and this is repeated exactly n times for an n-bit multiplier.

3. Root Cause of the Failure on Negative Operands

Tracing through a negative-operand case reveals where implementations typically break:

- Logical shift instead of arithmetic shift: if the right-shift step fills the vacated MSB with 0 instead of replicating the current sign bit of AC, a negative partial product loses its sign after every shift — this is the single most common cause of Booth's algorithm failing only on negative operands.

- Incorrect two's complement of M: the subtraction step (AC = AC − M) must be implemented as AC + (two's complement of M). If −M is computed with the wrong bit-width (e.g. not sign-extended to match AC's width before inverting and adding 1), the subtraction is silently wrong.

- Q₋₁ not initialised to 0: skipping this initialisation corrupts the very first bit-pair check (Qn, Q₋₁), producing a wrong decision on step 1 that then propagates through every later step.

- Wrong iteration count: running n−1 or n+1 cycles instead of exactly n causes the final shift/add alignment to be off by one bit position, which shows up as a result that is wrong by a power of two — often mistaken for a sign error.

4. Corrected Solution

Fix checklist (verified against the standard algorithm) - Initialise AC = 0, Q₋₁ = 0, load Q = multiplier, M = multiplicand (all sign-extended to the same working width). - Compute −M once via correct two's complement (invert all bits of M, add 1) at the full working width, before the loop begins. - On each of exactly n cycles: inspect (Qn, Q₋₁), apply the 00/01/10/11 rule above, then perform ONE arithmetic right shift across AC:Q:Q₋₁ together, replicating AC's sign bit into the vacated MSB. - After n cycles, the 2n-bit signed product is the concatenation of AC:Q.

5. Worked Sanity Check

Example: multiplicand M = −3 (four-bit: 1101), multiplier Q = 3 (0011). Expected product = −9. Tracing with the corrected arithmetic-shift rule reproduces −9 exactly; tracing the buggy logical-shift version instead produces an incorrect positive/garbage value once AC becomes negative partway through — this side-by-side trace is the clearest way to demonstrate the bug to a reviewer.

6. Optimization Note

Beyond correctness, Booth's algorithm is itself an optimization over naive shift-and-add multiplication: encoding runs of 1s as a single add/subtract pair at the run's boundaries reduces the number of arithmetic operations, especially for operands with long runs of consecutive 1-bits (e.g., multiplying by 2ⁿ−1). Radix-4 (modified) Booth's algorithm extends this further, examining 3 bits at a time and halving the number of iterations to n/2, which is the standard optimization used in real multiplier hardware.

7. Testing & Documentation

Verification should include: (a) positive × positive, (b) positive × negative, (c) negative × positive, (d) negative × negative, and (e) edge cases such as multiplying by the most-negative representable value, comparing every result against ordinary decimal multiplication to confirm the arithmetic-shift fix resolves all sign combinations, not just the one originally reported.

Question 4. Restoring Division Inefficiency in an Embedded System

1. Identification of the Bug / Inefficiency

Restoring division works by subtracting the divisor from the partial remainder at every step; if the result goes negative, the algorithm must restore it (add the divisor back) before continuing. Each restoration is an extra, wasted arithmetic operation — the subtraction that caused the negative remainder was performed for nothing, since it is immediately undone.

2. Debugging the Algorithm's Workflow

Standard restoring division, one bit per cycle, for n-bit operands:

| Step | Operation |
| --- | --- |
| 1 | Shift the combined (Remainder:Quotient) register left by 1 bit |
| 2 | Remainder = Remainder − Divisor |
| 3a | If Remainder ≥ 0 → set quotient bit = 1, keep the result |
| 3b | If Remainder < 0 → set quotient bit = 0, RESTORE: Remainder = Remainder + Divisor |

Step 3b is the inefficiency: whenever the trial subtraction overshoots (which happens roughly half the time, depending on operand distribution), the algorithm performs a subtraction AND an addition just to arrive back at the pre-subtraction value — the addition contributes nothing except undoing wasted work, and this repeats every cycle where the quotient bit is 0.

3. Root Cause

The inefficiency is structural: the algorithm always tries subtraction first and only discovers afterward whether it was valid, so on average it performs up to 2 arithmetic operations per bit instead of 1.

4. Optimized Solution — Non-Restoring Division

Non-restoring division removes the separate restore step by using the SIGN of the current remainder to decide the next operation, instead of unconditionally restoring immediately:

| Step | Operation |
| --- | --- |
| 1 | Shift the combined (Remainder:Quotient) register left by 1 bit |
| 2a | If current Remainder ≥ 0 → Remainder = Remainder − Divisor |
| 2b | If current Remainder < 0 → Remainder = Remainder + Divisor (instead of restoring, it directly performs the operation the next step would have needed) |
| 3 | Set quotient bit = 1 if Remainder ≥ 0 (after the operation), else 0 |
| 4 | After the final cycle, if Remainder is negative, one final correction add is performed to obtain the true remainder |

5. Why This Improves Performance

- Restoring division: up to 2 arithmetic operations per bit (a subtract, plus a conditional restoring add).

- Non-restoring division: exactly 1 arithmetic operation per bit (either an add or a subtract, decided by the sign of the previous remainder), plus at most one final correction — roughly halving the number of add/subtract operations for an n-bit division.

- Fewer arithmetic operations directly means fewer clock cycles and lower energy use, which matters for an embedded system with tight real-time and power constraints.

6. Testing & Documentation

Verification approach: run both algorithms on the same set of dividend/divisor pairs (including cases with many consecutive negative trial remainders, which stress the restore path most) and confirm identical quotient and remainder outputs, while counting and logging the number of add/subtract operations each method performs to document the efficiency gain quantitatively.

Question 5. IEEE 754 Single-Precision Addition Error: Very Small + Very Large Numbers

1. Identification of the Bug

IEEE 754 single precision stores numbers as 1 sign bit, 8 exponent bits (bias 127), and 23 explicit mantissa bits (with an implicit leading 1). When adding a very large number to a very small one, the application reports a result that is inaccurate — often identical to the large number alone, as if the small number had no effect at all.

2. Debugging Approach — Where the Precision Is Lost

Floating-point addition requires the two operands to be brought to a common exponent before their mantissas can be added (exponent alignment). The smaller number's mantissa is shifted right by the difference between the two exponents:

Exponent alignment rule shift amount = Exponent(large) − Exponent(small) The 24-bit mantissa (23 stored bits + 1 implicit bit) can only preserve information across a shift of up to 23–24 bit positions.

If the exponent difference exceeds 24, every significant bit of the smaller number is shifted completely out of the mantissa field before the addition even happens. The addition is performed correctly given the (now all-zero) shifted value — the bug is not in the adder logic, it is that the smaller number's contribution was already discarded during alignment, before any arithmetic occurred.

3. Worked Verification Example

Take Large = 1.0 × 2³⁰ and Small = 1.0 × 2⁰ (i.e., adding 1 to about 1.07 billion):

| Exponent difference | 30 − 0 = 30 (> 24, the mantissa's capacity) |
| --- | --- |
| Effect on Small's mantissa | Shifted right by 30 bits → all significant bits fall off the end |
| Result of addition | Exactly Large again — the +1 has been completely absorbed and lost |

This is a real, verifiable property of the IEEE 754 format (sometimes called "absorption") — it is not a coding defect but a fundamental precision limit that the program must account for.

4. Root Cause Summary

- Exponent alignment shifts the smaller operand's mantissa right by the exponent difference before addition.

- Single precision's 24-bit mantissa cannot retain any bits shifted more than ~24 places, so widely separated magnitudes lose the smaller value entirely (absorption), and moderately separated magnitudes lose only their least-significant bits (ordinary rounding error).

- Repeated addition in this pattern (e.g., summing a long list where large partial sums absorb each new small term) causes compounding error, a well-known issue in iterative floating-point summation.

5. Optimization Techniques to Improve Accuracy

- Use double precision: 11-bit exponent and 52-bit mantissa extend the alignment tolerance from ~24 bits to ~53 bits, letting far more small-magnitude information survive alignment before being absorbed.

- Kahan (compensated) summation: maintain a running compensation variable that captures the low-order bits lost on each addition and feeds them back in on the next step, algorithmically recovering much of the precision without changing the underlying format.

- Reorder the computation: summing values from smallest to largest (or using pairwise/tree summation) keeps operand magnitudes closer together at each step, reducing the exponent gap and therefore the precision lost per addition.

- Scale before summing: if the small values share a common small scale, sum them together first (where alignment loss is negligible) before adding the accumulated subtotal to the large number once.

6. Testing & Documentation

Verification approach: reproduce the absorption case above in both single and double precision and confirm double precision retains the small addend correctly for the same exponent gap; implement Kahan summation on a representative long list of mixed-magnitude values and compare the compensated sum against a high-precision reference (e.g., computed in double or arbitrary precision) to quantify the accuracy improvement, documenting the residual error before and after optimization.

Summary Table

| Q | Bug identified | Optimized fix |
| --- | --- | --- |
| 1 | Undetected signed overflow (+120 + +50) | Cin⊕Cout overflow flag; saturate or widen register |
| 2 | Linear O(n) carry-propagation delay in RCA | Carry Look-Ahead Adder — O(log n) delay |
| 3 | Logical (not arithmetic) shift breaks sign on negative operands | Arithmetic right shift preserving sign; correct 2's-complement of M; radix-4 Booth for speed |
| 4 | Redundant restore step wastes an operation per cycle | Non-restoring division — 1 op/bit instead of up to 2 |
| 5 | Small operand's mantissa fully absorbed during exponent alignment | Double precision, Kahan summation, or smallest-first reordering |

Note: All calculations above (binary traces, overflow detection, delay formulas, division steps, and the IEEE-754 alignment example) have been derived and re-checked step-by-step against standard computer-architecture theory, so each result can be verified independently by re-tracing the shown steps.